PUBLIC SAFETY COLLINEAR OMNI ANTENNA 746-869 MHz

927-70-7.0-43F

The Null Fill is optional, the Down tilt degrees available from 1 to 10 in one degree increment.

| Electrical Specifications | 927-70-7.0-43F |  |  |
| --- | --- | --- | --- |
| Frequency Range | 740-870 MHz |  |  |
| Gain | 7.0 dBd |  |  |
| VSWR (Return Loss) | <1.5:1 (14dB) |  |  |
| Passive IM 3rd Order (2x20W) | -150 dBc |  |  |
| Peak Instantaneous Power (PIP) | 25 kW |  |  |
| Power Input | 500 W |  |  |
| Vertical Beamwidth | 12° |  |  |
| Horizontal Beamwidth | Omni +/-0.25 dB |  |  |
| Input | 4.3-10 Female |  |  |
| Beam Tilt | 0-10 degree options |  |  |
| Mechanical Specifications | 927-70-7.0-43F |  |  |
| Configuration |  | Corporate Fed Dipole Array |  |
| Length |  | 84” |  |
| Radome Diameter |  | 2.5” |  |
| Weight |  | 18 lbs |  |
| Shipping Weight |  | 26 lbs |  |
| Shipping Dimensions | L | 86” |  |
|  | W | 4” |  |
|  | H | 4” |  |
| Mounting Area |  | 12.0” x 3.0” diam. aluminum |  |

PUBLIC SAFETY COLLINEAR OMNI ANTENNA 740-870 MHz

Environmental Specifications                      927-70-7.0-43F

927-70-7.0-43F

927-70-7.0-43F

927-70-7.0-43F

IP Rating	66

Projected Area	0.88 ft2 (no ice)

1.21 ft2 (with ice)

Lateral Thrust @ 100mph	35 lbs.

Wind Gust Rating	150 mph

Standard Elevation Pattern

PUBLIC SAFETY COLLINEAR OMNI ANTENNA 740-870 MHz

Null Fill